Cabled-16_AT50-41_Discrete_Summary Information:

File Mapping:

Hex files were renamed from the original ship-provided files for consistency and ease of processing.  Original file names are listed below on the left, with corresponding new file names on the right.  Original Hex files are accessible for each cruise in the Cruise Data folder under the “Ship Data” sub-directory.  Bottle files used to populate the discrete summary can be found in the Water Sampling sub-directory under “Shipboard Data”.

at5041_001 = AT50-41_CTD-001

J2-1696_CT2 = AT50-41_J2-1696

J2-1698_CT2 = AT50-41_J2-1698

at5041_002 = AT50-41_CTD-002

J2-1699_CT2 = AT50-41_J2-1699

at5041_003 = AT50-41_CTD-003

J2-1702_CT2 = AT50-41_J2-1702

J2-1704_CT2 = AT50-41_J2-1704

at5041_004 = AT50-41_CTD-004

J2-1706_CT2 = AT50-41_J2-1706

at5041_005 = AT50-41_CTD-005

J2-1708_CT2 = AT50-41_J2-1708

J2-1713_CT2 = AT50-41_J2-1713

J2-1714_CT2 = AT50-41_J2-1714

J2-1719_CT2 = AT50-41_J2-1719

J2-1723_CT2 = AT50-41_J2-1723

at5041_007 = AT50-41_CTD-007

J2-1735_CT2 = AT50-41_J2-1735

J2-1736_CT2 = AT50-41_J2-1736

at5041_008 = AT50-41_CTD-008

at5041_009 = AT50-41_CTD-009

at5041_010 = AT50-41_CTD-010

J2-1741_CT2 = AT50-41_J2-1741

J2-1743_CT2 = AT50-41_J2-1743

J2-1696_sealogExport = AT50-41_J2-1696_sealogExport

J2-1698_sealogExport = AT50-41_J2-1698_sealogExport

J2-1699_sealogExport = AT50-41_J2-1699_sealogExport

J2-1702_sealogExport = AT50-41_J2-1702_sealogExport

J2-1704_sealogExport = AT50-41_J2-1704_sealogExport

J2-1706_sealogExport = AT50-41_J2-1706_sealogExport

J2-1708_sealogExport = AT50-41_J2-1708_sealogExport

J2-1713_sealogExport = AT50-41_J2-1713_sealogExport

J2-1714_sealogExport = AT50-41_J2-1714_sealogExport

J2-1723_sealogExport = AT50-41_J2-1723_sealogExport

J2-1735_sealogExport = AT50-41_J2-1735_sealogExport

J2-1736_sealogExport = AT50-41_J2-1736_sealogExport

J2-1741_sealogExport = AT50-41_J2-1741_sealogExport

J2-1743_sealogExport = AT50-41_J2-1743_sealogExport

Summary Notes:

AT50-41, J2-1698, Niskin Forward: DiscreteSampleFlag: Chlorophyll sample lost before filtration

AT50-41, CTD-002, Niskin 7: DiscreteSampleFlag: Oxygen sample contained floating particles (purple)

AT50-41, CTD-002, Niskin 21: DiscreteSampleFlag: Oxygen sample contained floating particles (purple)

AT50-41, CTD-004, Niskin 10: DiscreteSampleFlag: Oxygen sample may contained double acid

AT50-41, J2-1723, Niskin Forward: DiscreteSampleFlag: Chlorophyll sample filtered 10 days after collection

AT50-41, CTD-010, Niskin 7: DiscreteSampleFlag: Oxygen bottle cap mismatch, labeled as "RCA-045"

AT50-41, CTD-010, Niskin 16: DiscreteSampleFlag: Oxygen titration possibly overshot

AT50-41, J2-1741, Niskin Aft: DiscreteSampleFlag: Oxygen samples pickled with miscalibrated/flooded pipette; Standards done with un-calibrated pipette

AT50-41, J2-1741, Niskin Aft: DiscreteSampleFlag: Oxygen samples pickled with miscalibrated/flooded pipette; Standards done with un-calibrated pipette

AT50-41, J2-1743, Niskin Aft: DiscreteSampleFlag: Oxygen samples pickled with miscalibrated/flooded pipette; Standards done with un-calibrated pipette

AT50-41, J2-1743, Niskin Aft: DiscreteSampleFlag: Oxygen samples pickled with miscalibrated/flooded pipette; Standards done with un-calibrated pipette

AT50-41, CTD-003, water collected for HPIES only

AT50-41, CTD-005, water collected for grad student Lacie Levy only

AT50-41, J2-1719, water collected for grad student Lacie Levy only

AT50-41, CTD-007, CTD-006 aborted because R/V Atlantis was directly over shallow profiler, not 500m downcurrent

AT50-41, CTD-009, water collected for SeapHOx equilibration

General File Notes:

- Discrete sample fields containing text or non-decimal numbers (“DIC-###”, “CH-##”, “OX-###”, “SA####”, or “673”) list sample bottle numbers and not analyzed data. Bottle numbers are included when data are not yet available, and will be replaced with analysis results as data are received from analysis labs.

- Fill value = -9999999

- Carbon analysis was provided by Burke Hales lab at Oregon State University.  All Carbon parameters are provided at in situ temperatures.  Calculated carbon parameters were provided by Hales using custom software routines using published values for the various carbon chemistry constants.  Hale’s lab provides the following references regarding constants used in the calculations.  For further information, please contact Burke Hales (bhales@coas.oregonstate.edu).

  - Carbonic acid dissociation constants: Millero (2010), with full resolution constants provided by Millero via private conversation, equal to Lueker’s constants at S > 25).

  - Kw: Millero (1995)

  - Kb: Dickson (1990)

  - Ksp for calcite and aragonite: Mucci (1980)

  - Kh: Weiss (1973)

  - Alkalnity is modeled as: HCO3- + 2CO3= + B(OH)4- + OH- - H+

Data Flag bit maps:

| Bit Position | Cast Flags | CTD File Flags | CTD Parameter Flags | Niskin Flags | Discrete Sample Flags | Discrete Replicate Flags |
| --- | --- | --- | --- | --- | --- | --- |
| 0 | Notes/Other | Notes/Other | Notes/Other | Notes/Other | Notes/Other | Notes/Other |
| 1 | Delayed start to data collection | Data cast only, no Niskins triggered | Not calibrated | Bottle information unavailable | Sample for this measurement was drawn from water bottle but analysis not received | Duplicate analysis on same sample |
| 2 | Acceptable; normal cast according to SOP | Acceptable; file processed according to SOP | Acceptable measurement | No problems noted | Acceptable; sample processed according to SOP | Single sample |
| 3 | Non-standard winch speed | File processed using modified parameters | Questionable measurement | Leaking | Questionable measurement | Duplicate analysis from same Niskin |
| 4 | Non-standard surface soak time | File processed using alternate XMLCON | Bad measurement | Ran out of water during sampling | Bad measurement | Triplicate analysis from same Niskin |
| 5 | Non-standard bottle soak time before Niskin trip | Missing scans as indicated by modulo error counts | Not reported | Vent open | Not reported | Unassigned |
| 6 | Sensor issues but cast completed and data collected | Missing metadata | Calibration coefficients > 1 year old | Misfire at wrong depth | Sample collected out of order | Unassigned |
| 7 | Cable issues but cast completed and data collected | Unassigned | Corresponding discrete sample | Unknown problem | Sample processed using alternative method; see notes | Unassigned |
| 8 | Winch issues but cast completed and data collected | Unassigned | Unassigned | Unassigned | Unassigned | Unassigned |
| 9 | Premature cast end with data and/or data loss | Unassigned | Unassigned | Sample not drawn for this measurement from this bottle | Unassigned | Unassigned |
| 10 | Significant ship heave | Unassigned | Unassigned | Unassigned | Unassigned | Unassigned |
| 11 | Station position not adequately maintained during cast | Unassigned | Unassigned | Unassigned | Unassigned | Unassigned |
| 12 | Tow-yo, Yo-yo cast | Unassigned | Unassigned | Unassigned | Unassigned | Unassigned |
| 13 | ROV bottle sample | Unassigned | Unassigned | Unassigned | Unassigned | Unassigned |
| 14 | Unassigned | Unassigned | Unassigned | Unassigned | Unassigned | Unassigned |
| 15 | Unassigned | Unassigned | Unassigned | Unassigned | Unassigned | Unassigned |